CS-358 project: Sphere

A self-contained spherical robot inspired by BB-8. CS-358 final project, EPFL.

# Project overview

This project was developed as part of the CS-358 course at EPFL, with a strong emphasis on real-world applicability, modular hardware/software design, and an engaging user interface.

## About the robot

It is a self-contained spherical robot inspired by BB-8 from Star Wars. A 3D-printed shell rolls on the ground, driven from the inside by a small chassis with three omnidirectional wheels. On top of the sphere, a separate "head" is held only by magnets, so it can rotate freely as the sphere moves. The head carries a camera.

The robot:

- Rolls in any direction using 3 omni wheels at 120° spacing, tilted inward at 30° against the inside of the sphere.

- Is driven over Wi-Fi from a browser dashboard.

- Streams live video from the head camera (planned — head firmware is in progress).

- Self-stabilizes with a 9-DOF IMU and a 100 Hz control loop.

- Can be reflashed over Wi-Fi (OTA), so the sphere doesn't have to be opened.

# List of materials

## Electronic components

- Microcontroller (body) : ESP32 Wemos D1 R32 / UNO32. Runs the control loop and the WebSocket server.

- Camera (head) : XIAO Vision AI Camera from Seeed Studio.

- Motors (×3) :  DFRobot FIT0186. 12 V DC gear motors with encoders, one per omni wheel.

- Motor drivers (×3).  L298N H-bridge modules. One channel per motor.

- IMU :  Adafruit BNO055. 9-DOF with built-in sensor fusion. I²C address 0x28.

- Battery : LiPo, [TODO: capacity/voltage].

- BMS : [TODO: model]. Protects the LiPo.

- Buck converter : LM2596 (or equivalent). 12 V → 5 V for the ESP32.

- Magnets : Neodymium, [TODO: dimensions]. Hold the head onto the sphere.

Hardware wiring diagram:

## Mechanical parts

- Sphere shell, 35 cm diameter : 3D printed in PETG. Two hemispheres, joined with magnets and tape.

- Internal chassis : laser-cut MDF. Holds the 3 motors at 120°, tilted inward by 30°.

- Omni wheels (×3), ~9.25 cm diameter — 3D printed in PETG.

- Head shell : 3D printed in PETG. Holds the camera and magnets.

- Head-to-sphere contact : 3D-printed bearing cages with metal ball-bearings.

# Build the robot

## Sphere shell

The sphere is what the wheels push against, so it needs to be round, rigid, and smooth on the inside. Inner radius is ~17.5 cm (35 cm diameter), matching the ROBOT_RADIUS constant in the firmware.

- 3D print the two hemispheres (top and bottom) in PETG on a large-format printer (e.g. Prusa XL).

- [TODO: slicer settings — layer height, walls, infill, supports]

- Sand the inside of each hemisphere until it feels smooth. This is what gives the wheels good grip.

- Only join the two hemispheres at the very end, once the chassis is in place inside.

## Internal chassis

The chassis holds the three motors at 120° spacing, each tilted inward at 30° from vertical so the wheels press against the inside of the sphere.

- Laser-cut the chassis pieces from MDF.

- Assemble with screws. Test-fit the three motors first — don't glue or screw anything down until they fit cleanly.

- Mount the motors at 0°, 120°, and 240° looking from above (front, back-right, back-left).

- Mount the IMU as far as possible from the head magnets, and as low as possible in the chassis. A low center of mass keeps the robot stable.

## Motors and wheels

- Print the 3 omni-wheel assemblies in PETG. Check that the sub-rollers spin freely.

- Press-fit each wheel onto the motor's D-shaft.

- Check the rolling direction. Motor 0 is at the front, motor 1 back-right, motor 2 back-left. If a motor spins the wrong way, swap fwd/rev in body-firmware/src/config/pins.h rather than rewiring.

## Electronics assembly

### Motor drivers (L298N)

| Motor | ESP32 GPIO (forward) | ESP32 GPIO (reverse) |
| --- | --- | --- |
| 0 (front) | 14 | 27 |
| 1 (back-right) | 16 | 17 |
| 2 (back-left) | 25 | 26 |

Jumper EN HIGH on all three L298N boards. Connect L298N GND to ESP32 GND.

### Encoders

| Motor | Encoder A | Encoder B |
| --- | --- | --- |
| 0 | 34 | 35 |
| 1 | 2 | 4 |
| 2 | 36 | 39 |

### IMU (BNO055)

| BNO055 | ESP32 |
| --- | --- |
| VIN | 3.3 V |
| GND | GND |
| SDA | 21 |
| SCL | 22 |

I²C address 0x28.

### Power

LiPo ──┬── BMS ──┬── 12 V ──── L298N supply (×3)

│         │

│         └── LM2596 ── 5 V ── ESP32

│                            └── BNO055 + encoders (via ESP32 3.3 V)

└── status LED

Add bulk capacitance (≥ 1000 µF + 0.1 µF ceramic) on the 12 V rail near each L298N. Without it, the motors will brown out the ESP32 every time you accelerate.

## Head and camera

The head houses the XIAO Vision AI Camera and its own small LiPo. It joins the same Wi-Fi as the body but streams video separately.

- 3D print the head shell.

- Mount the camera with the lens facing forward.

- Wire the head LiPo through its own BMS and buck converter to the camera's 5 V input.

[TODO: photo of the head opened]

## Magnetic head mount

The head holds onto the sphere with magnets only,  there is no mechanical link. That's what lets the head swivel independently while the sphere rolls.

- Press neodymium magnets into the head's underside, alternating polarities.

- Press matching magnets onto the top of the chassis (which sits inside the sphere, just under the head).

- Metal ball-bearings in 3D-printed cages provide low-friction contact between the head and the top of the sphere.

# Algorithms

## Omni-wheel inverse kinematics

Given a desired body velocity (vx, vy, ω), we compute each wheel's target speed using the geometry of a 120°-spaced, inward-tilted omni layout.

For a wheel at angle θᵢ from the front:

ωᵢ = (1 / (r · cos α)) · [ −sin(θᵢ)·vx − cos(θᵢ)·vy − R·ω ]

where r is the wheel radius, R is the distance from the chassis center to the wheel contact, and α is the wheel tilt.

Our values (from RobotConstants.h):

- r = 0.046225 m (wheel radius — 9.245 cm wheel diameter)

- R = 0.1745 m (robot radius — center to wheel contact)

- α = 30° (wheel tilt from vertical)

- MAX_RPM = 251 (per-wheel saturation)

For our layout (θ₀ = 0°, θ₁ = 120°, θ₂ = 240°):

1            ┌  0     −1     −R  ┐   ┌ vx ┐

[ωᵢ] = ───────────  ·    │ −√3/2   1/2  −R   │ · │ vy │

r · cos(α)       └  √3/2   1/2  −R   ┘   └  ω ┘

Quick sanity checks:

- Pure forward → motor 0 doesn't spin, motors 1 and 2 spin opposite ways. ✓

- Pure rotation → all three spin in the same direction at the same speed. ✓

- Flat layout (α = 0) → equations match the standard 3-omni textbook case. ✓

Full derivation in code/body-firmware/docs/omni-kinematics-derivation.md.

## Per-wheel PID

Each motor has its own PID loop tracking the target speed from the kinematics.

- Input: target RPM from kinematics

- Feedback: actual RPM from the encoder

- Output: PWM duty cycle (−1.0 to +1.0)

- Gains: Kp = 0.005, Ki = 0.002, Kd = 0.0005 (integrator clamped to ±5.0)

- Loop rate: 100 Hz

Code in code/body-firmware/src/control/PID.h.

## Tilt-limit controller

The motors are powerful enough that on hard acceleration the chassis can climb the inside of the sphere and flip over. To prevent this, we designed a controller that watches the IMU:

- Active (tilt < 15°) — normal driving.

- Recovery (tilt 15–20°) — coast the motors, ignore the joystick, wait for gravity. Re-engages once tilt drops by ~3°.

- Stopped — brake all motors, reset PIDs.

- Hard cap (tilt > 30°) — brake immediately, regardless of mode.

Status: designed (see controller-discussion.md) but not yet integrated. The current shipping controller is a simple pass-through (PassthroughDrivetrainController), which is fine on flat ground with a careful operator.

# Control architecture

## How the two ESP32 cores split the work

- Core 0 runs the WebSocket server. It receives vx,vy,omega text frames from the laptop and drops the latest one into a one-slot mailbox (old commands get overwritten, never queued).

- Core 1 runs the 100 Hz control loop. It reads the latest command, reads the IMU, runs the kinematics and the PIDs, and updates the motors.

The pipeline looks like this:

WebSocket frame → mailbox → control loop (100 Hz) → kinematics → 3 × PID → 3 × motor

## Safety properties

- Stale commands stop the robot. If no fresh command arrives within 200 ms, the motors ramp smoothly to zero. No jerk, no runaway.

- Watchdog reboot. If the control loop ever hangs, the ESP32 resets itself within 1 second.

- Wi-Fi watchdog. If Wi-Fi has been disconnected for more than 30 s, the ESP32 reboots to reconnect cleanly.

- Safe OTA. Flashing over Wi-Fi first stops accepting commands, so the ramp-to-zero takes effect before the firmware is overwritten.

- Latest command wins. A joystick command from a second ago is worse than no command, so we never queue, we always use the most recent frame.

# Communication protocol

The laptop talks to the body ESP32 over WebSocket on the local Wi-Fi.

- The ESP32 is the server, listening on port 80.

- The webapp is the client.

## Command format

Plain text, comma-separated:

"vx,vy,omega"

| Field | Unit | Meaning |
| --- | --- | --- |
| vx | m/s | Forward (positive = forward) |
| vy | m/s | Strafe (positive = left) |
| omega | rad/s | Yaw rate (positive = counterclockwise from above) |

Extra fields at the end are ignored — that's how we'll add new fields later (e.g. head_tilt) without breaking older clients.

## Video stream

The head camera streams video to a small relay server (code/stream-client/relay.ts) running on the laptop. The webapp subscribes to the relay to display the feed.

- Camera → ws://<laptop-ip>:3005/?role=producer

- Browser → ws://<laptop-ip>:3005/

The relay just fans out every frame from the camera to every viewer.

# Web app

[TODO: paste a screenshot of the webapp dashboard]

## What you can do from the dashboard

- Live camera feed — real-time video from the head.

- Camera status — online / offline / reconnecting.

- Manual control — joystick or keyboard input.

- Emergency stop — immediately sends 0,0,0 and locks input until you resume.

- Telemetry — IMU pose, battery voltage, loop rate.

The webapp is a Next.js app on port 3003. The video relay is a small Node server on port 3005. The video viewer is another Next.js app on port 3004.

Before running it, set your Wi-Fi credentials in code/body-firmware/src/config/wifi_credentials.h.

# Code structure

code/

├── Makefile                  ← `make help` lists every target

├── make/                     ← split Makefiles (setup, web, firmware)

├── body-firmware/            ← ESP32 firmware (PlatformIO + C++17)

│   ├── platformio.ini        ← 6 envs: wemos_d1_uno32, drivetrain_test,

│   │                            motor_test, robot, robot_ota, native

│   ├── docs/                 ← test procedures, kinematics, design specs

│   ├── src/

│   │   ├── main_robot.cpp            ← production entry point

│   │   ├── main_motor_test.cpp       ← per-motor bring-up

│   │   ├── main_drivetrain_test.cpp  ← full kinematics over serial

│   │   ├── main_imu.cpp              ← BNO055 bring-up

│   │   ├── main_blink.cpp            ← LED life check

│   │   ├── config/                   ← pins.h, debug.h, wifi_credentials.h

│   │   ├── motor/                    ← motor + driver classes

│   │   ├── drivetrain/               ← kinematics + drivetrain + controllers

│   │   ├── imu/                      ← BNO055 wrapper

│   │   ├── input/                    ← WebSocket producer + frame parser

│   │   ├── control/                  ← PID

│   │   └── util/                     ← math + sync primitives

│   └── test/                 ← native unit tests (fff mocking library)

├── head-firmware/            ← head MCU firmware (in progress)

├── webapp/                   ← Next.js operator UI (port 3003)

└── stream-client/            ← Next.js video viewer (port 3004) + relay (port 3005)

## Firmware libraries

From body-firmware/platformio.ini:

- adafruit/Adafruit BNO055 @ 1.6.4

- adafruit/Adafruit Unified Sensor @ 1.1.15

- adafruit/Adafruit BusIO @ 1.17.4

- thijse/ArduinoLog @ 1.1.1

- madhephaestus/ESP32Encoder @ 0.12.0

- links2004/WebSockets @ 2.7.3

# Setup

## Prerequisites

- Node.js ≥ 18

- pnpm (npm install -g pnpm)

- Python 3 + pip3

- PlatformIO Core (installed automatically by make setup-firmware if missing)

- A Wi-Fi network the robot and your laptop can both join

## Step 1 — Install dependencies

cd code

make setup

This runs three sub-targets: setup-prereqs (system tools), setup-web (pnpm install), and setup-firmware (PlatformIO + the C++ libraries listed above).

## Step 2 — Configure Wi-Fi

cp code/body-firmware/src/config/wifi_credentials.example.h \

code/body-firmware/src/config/wifi_credentials.h

Edit the new file and set WIFI_SSID, WIFI_PASSWORD, and OTA_PASSWORD (any password — you'll need the same one for OTA updates later).

## Step 3 — Connect the board

Plug the Wemos D1 UNO32 into your computer with a data-capable USB cable (charge-only cables won't work).

## Step 4 — Flash the firmware

cd code/body-firmware

pio run -e robot -t upload

If the upload fails to connect, hold the BOOT button on the board while PlatformIO retries, and release it once the upload starts.

Confirm boot:

pio device monitor -b 115200

You should see:

BB-8 main_robot starting.

Connecting to WiFi....

WiFi connected. IP: 192.168.x.x

[ota] ready on bb8-robot.local

BB-8 ready.

## Step 5 — Flash future updates over Wi-Fi

Once the sphere is sealed, USB is annoying. Use OTA:

export OTA_PASSWORD='whatever-you-set'

pio run -e robot_ota -t upload

If .local doesn't resolve on your network, use the raw IP:

pio run -e robot_ota -t upload --upload-port=<robot-ip>

# How to run the robot

- Power on the body by connecting the LiPo to the BMS. The boot log should print "BB-8 ready." once Wi-Fi is up.

- Power on the head (it has its own battery).

- Start the laptop services:

cd code

make dev

This launches the webapp (port 3003), the video viewer (port 3004), and the relay (port 3005).

- Open http://localhost:3003 in your browser.

- Open http://localhost:3003/video to confirm the camera is streaming.

- Drive!

# Testing & validation

Each subsystem has its own PlatformIO build target so it can be tested on its own before running the full robot. Note that main_blink.cpp and main_imu.cpp are compiled in the same envs (drivetrain_test / motor_test exclude them; the default env compiles main_imu.cpp), so the order below maps to environments, not to a separate "blink" target.

Recommended bring-up order:

- Per-motor (open loop) : pio run -e motor_test -t upload. Drive each motor at low PWM and verify the encoder reads back consistent counts.

- IMU :  pio run -e wemos_d1_uno32 -t upload (this env compiles main_imu.cpp by default). Calibrate the BNO055, check axis orientation.

- Drivetrain (closed loop) :  pio run -e drivetrain_test -t upload. Full kinematics over serial, with the wheels off the ground.

- Full robot : pio run -e robot -t upload. Start on carpet.

Test procedures with pass/fail checklists are in code/body-firmware/docs/:

- motor-spin-test-procedure.md

- imu-test-procedure.md

- drivetrain-test-procedure.md

For host-side unit tests (using the fff mocking library):

make test                # everything

make test-firmware       # PlatformIO native tests

make test-webapp         # webapp tests

make test-stream         # stream-client tests

# Safety

The full safety procedure is in documents/BB8_Safety_Procedures.pdf. Read it before every test session, not just the first one.

## LiPo charging

Separate guide: see the LiPo charging PDF in documents/. Short version: never charge unattended, never charge a swollen or hot LiPo, always charge inside a LiPo-safe bag.

## Magnet safety

The head magnets are strong. Keep them away from credit cards, hotel keys, hard drives, pacemakers, medical implants, and mechanical watches. Mind your fingers when handling — they can pinch hard.

# Troubleshooting

## Electrical

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| ESP32 brown-outs when motors start | No bulk cap on 12 V rail, or under-rated buck | Add ≥ 1000 µF on 12 V; check the buck is rated ≥ 2 A peak |
| Motor 1 behaves erratically | GPIO 16/17 conflict with PSRAM (WROVER only) | Use a WROOM-based ESP32 — the Wemos D1 UNO32 is fine |
| Encoder reads 0 while motor spins | Encoder A/B disconnected or swapped | Re-seat the wires; swap A/B in pins.h |
| Encoder RPM sign is reversed | Encoder A/B swapped | Swap A and B for that motor in pins.h |
| Serial shows nothing | Wrong baud rate, or charge-only USB cable | 115200 baud; data-capable cable |
| IMU prints "BNO055 not detected" | SDA/SCL swapped, wrong address, or magnet interference | Check wiring (SDA=21, SCL=22); confirm 0x28; move magnets away |

## Firmware

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Upload fails to connect | ESP32 not in download mode | Hold BOOT while PlatformIO connects |
| OTA can't find bb8-robot.local | mDNS not resolving on your OS | Use the raw IP: --upload-port=<robot-ip> |
| Robot rotates instead of going forward | Motor numbering doesn't match physical layout | Re-label the motors or update pins.h |
| Robot oscillates / buzzes | PID Kp too high, or Kd too low | Lower Kp first |
| Robot ignores joystick after disconnect | Working as designed — stale commands stop the robot | Reconnect; send a fresh frame |

## Network / web

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Video feed is black | Relay or stream-client not running | Start each separately: make dev-relay, make dev-stream |
| Browser can't reach the robot | Different VLANs / Wi-Fi bands | Put both on the same SSID/band |
| Webapp port already in use | Leftover dev server | make dev-stop (kills ports 3003–3005) |

# Improvements

## Software

- Build out the RC Control page (currently a placeholder).

- Add a telemetry overlay to the webapp (IMU pose, battery voltage, loop rate).

- Finish the head firmware (video streaming, battery monitoring).

- Integrate the tilt-limit controller (currently designed but not yet shipping).

## Nice-to-haves

- Autonomous navigation (SLAM, obstacle avoidance) using the head camera.

- Face tracking — the XIAO Vision AI module has the CV chip for it.

- Voice interface (mic on the head, recognition off-board).

- Gesture-based control.

- Anti-vibration mount for the head camera.

# Team members

| Name | Email |
| --- | --- |
| Alessandro Lombardini | alessandro.lombardini@epfl.ch |
| Tristan Compain | tristan.compain@epfl.ch |
| Lucas Brunschwick | lucas.brunschwick@epfl.ch |
| Vincent du Fresne | vincent.dufresnevonhohenesche@epfl.ch |
| Alban Thèves | alban.theves@epfl.ch |

# Acknowledgements

References:

- Modern Robotics, Lynch & Park — Ch. 13.2, Omnidirectional Wheeled Mobile Robots

- Borisov et al., Nonholonomic dynamics and control of a spherical robot with an internal omniwheel platform (2016)

- SpheriDrive: A spherical robot with an innovative 3-wheeled platform (2025)

- Adam Green's BB-8 build

- BB-8 Builders Club

- ROS2 Mobile Robot Kinematics — Omnidirectional Wheels

- Voiture télécommandée par bluetooth — Les Fabriques du Ponant wiki